Day 01 Network Fundamentals & Wireshark Packet Analysis

Objective

The objective of this lab was to understand fundamental networking concepts including:

- OSI Model vs TCP/IP Model

- Unicast, Broadcast, and Multicast Communication

- TCP Communication

- ARP Resolution Process

- Packet Analysis using Wireshark

This lab provides foundational knowledge required for Cybersecurity, SOC Analysis, Network Monitoring, Incident Response, and Threat Hunting.

Tools Used

| Tool | Purpose |
| --- | --- |
| Wireshark | Network packet capture and analysis |
| Web Browser | Generate TCP traffic |
| Command Prompt | Generate ARP requests |
| Windows OS | Host system |

Lab Environment

| Component | Details |
| --- | --- |
| Operating System | Windows |
| Network Adapter | Wi-Fi |
| Packet Analyzer | Wireshark |
| Traffic Types Captured | ARP, TCP |

Task 1: ARP Broadcast Analysis

Objective

Capture and analyze ARP traffic to understand how devices discover MAC addresses on a local network.

Wireshark Filter

arp

Captured Traffic

Observed ARP requests and replies:

Who has 10.49.36.98? Tell 10.49.36.26

10.49.36.98 is at 70:cd:0d:82:66:30

Analysis

ARP (Address Resolution Protocol) is responsible for mapping an IP address to a MAC address within a local network.

The captured request packet was broadcast to all devices on the local network because the sender did not know the destination MAC address.

The device owning the requested IP address responded with its MAC address.

Communication Type

ARP Request

Broadcast

ARP Reply

Unicast

OSI Layer Mapping

| Protocol | OSI Layer |
| --- | --- |
| ARP | Layer 2 (Data Link Layer) |

Security Relevance

ARP traffic is frequently analyzed by SOC Analysts during:

- Network troubleshooting

- ARP Spoofing investigations

- Man-in-the-Middle attack detection

- Internal network monitoring

Observation

The ARP protocol successfully resolved the MAC address associated with the target IP address. The packet exchange demonstrated the broadcast nature of ARP requests and the unicast nature of ARP replies.

Task 2: TCP Traffic Analysis

Objective

Capture and analyze TCP traffic generated during communication with remote servers.

Wireshark Filter

tcp

Captured Traffic

Observed TCP packets including:

- TCP ACK

- TCP Keep-Alive ACK

- TCP Retransmission

- TCP Duplicate ACK

Traffic was observed between:

- Local Host: 10.49.36.98

Remote Hosts: Public Internet Servers

- Port: 443 (HTTPS)

Analysis

TCP (Transmission Control Protocol) is a connection-oriented protocol that provides reliable communication between devices.

The captured packets demonstrate:

ACK Packets

Used to acknowledge successful receipt of data.

Keep Alive Packets

Maintain active TCP sessions.

Retransmission Packets

Sent when previously transmitted packets are not acknowledged.

Duplicate ACK Packets

Indicate missing segments and trigger retransmission mechanisms.

Communication Type

Unicast

- TCP communication occurs directly between two endpoints.

OSI Layer Mapping

| Protocol | OSI Layer |
| --- | --- |
| IP | Layer 3 (Network Layer) |
| TCP | Layer 4 (Transport Layer) |
| HTTPS/TLS | Layer 7 (Application Layer) |

Security Relevance

TCP analysis is a critical skill for:

- SOC Analysts

- Incident Responders

- Threat Hunters

- Network Security Engineers

Common investigations include:

- Packet loss

- Network latency

- Suspicious communications

- Malware Command-and-Control traffic

- Data exfiltration detection

Observation

TCP traffic captured during HTTPS communication demonstrated reliable transport mechanisms through acknowledgements, retransmissions, and keep-alive packets. The traffic confirmed unicast communication between the client and remote servers.

OSI Layer Summary

| Protocol | Layer | Purpose |
| --- | --- | --- |
| Ethernet | Layer 2 | Frame delivery |
| ARP | Layer 2 | IP-to-MAC resolution |
| IP | Layer 3 | Routing |
| TCP | Layer 4 | Reliable transport |
| TLS/HTTPS | Layer 7 | Secure application communication |

Key Learning Outcomes

After completing this lab, I was able to:

- Capture and analyze network packets using Wireshark

- Differentiate between Broadcast and Unicast traffic

- Identify ARP requests and replies

- Analyze TCP communication patterns

- Map network protocols to their respective OSI layers

- Understand packet behavior relevant to SOC and cybersecurity operations

Conclusion

This lab provided practical exposure to network packet analysis using Wireshark. ARP traffic demonstrated Layer 2 address resolution mechanisms, while TCP traffic illustrated reliable transport-layer communication. These foundational concepts are essential for cybersecurity professionals working in Security Operations Centers (SOC), Incident Response, and Network Security Monitoring.